WEIGHLINK AUTOMATION SYSTEM (WAS)

ROBUST BLUETOOTH SCALES

- Mini Crane Scale (Generic/Various manufacturers)

  - Max Capacity: 500 kg (1100 lbs)

  - Min Weight (Readability/Division): Typically 0.1 kg or 0.2 kg

  - Type: Crane/Hanging Scale (portable for field use)

- CUBLiFT CS-500

  - Max Capacity: 500 kg (0.5 ton / 1000 lb)

  - Min Weight (Minimum Loading): 10 kg

  - Type: Crane/Hanging Scale

- HSCo PLCHQBT500 / PLSSWBT500

  - Max Capacity: 500 kg

  - Min Weight (Accuracy): 50g (0.05 kg) for some models, or 100g (0.1 kg) for others

  - Type: Platform Scale (requires a flat surface)

- Iscale ISP-500M

  - Max Capacity: 500 kg

  - Min Weight (Accuracy): 50g (0.05 kg)

  - Type: Platform Scale

ARCHITECTURE

Gateway device:

- Python script: reads weight (serial or BLE), parses value, local buffer (CSV/sqlite)

- Authenticates to Google Sheets API using service-account JSON

- Appends row(s) to the shared Google Sheet

End-users:

- Open Google Sheet on laptop / phone / tablet (live view)

- Excel can download CSV or use Sheets → Excel export

- (Optional) Data Studio/Looker studio dashboard reads the same sheet

Key behaviours:

Gateway immediately writes to Google Sheets if network available.

If network is down, gateway buffers locally (CSV or SQLite) and retries periodically.

Google Sheet is shared with the service-account email so the gateway can append rows.

Python Script generated and running on pydriod

Google Sheets setup (step-by-step)

1. Create a new Google Sheet; give it the name you used in `SPREADSHEET_NAME` (e.g., ` Weight_Logger`). In the sheet, optionally create columns: `ID|UTC Timestamp | Weight `.

2. Open Google Cloud Console and create a new Project (or use an existing one).

3. Enable the Google Sheets API and Google Drive API for the project.

4. Create a Service Account inside IAM & Admin → Service Accounts.

5. Create a JSON key for that service account and download. Put it into the same folder as the Python script.

6. Share the Google Sheet with the service account email ‘e.g., `my-service-account@project.iam.gserviceaccount.com` giving Editor access (File → Share).

7. Ensure `GOOGLE_CREDS_FILE` in the script matches the filename (e.g., `creds.json`) and `SPREADSHEET_NAME` matches the sheet title.

Setting Output up

On Windows (laptop)

1. Install Python 3.8+ from python.org and enable pip.

2. Install packages in PowerShell:

Powershell

pip install gspread oauth2client pyserial bleak requests

3. Pair the scale (Bluetooth pairing) so a Virtual COM port (e.g., COM4) appears in Device Manager.

4. Set `SERIAL_PORT = "COM4"` in the script, run:

Powershell

python scale_to_sheets.py

C. On Android (quick test / field)

Use Pydroid 3 (recommended) - install from Play Store.

Copy `scale_to_sheets.py` and `json file` to the phone storage (Pydroid folder).

Install required packages inside Pydroid (it has pip).

Connect via Bluetooth (Android may limit access to classic SPP from Python; often easier to use BLE).

Run script from Pydroid. (Android restrictions vary — if issues, use Pi or laptop as gateway.)